Solving a structure from a diffraction pattern (DP) is both easy, and rather hard. The hard part is that one has a series of decisions which need to be made, and getting good results depends upon making good enough choices. Specificically, what has to be considered is:

- What is the correct/best way to measure the data, i.e. what parameters to use for the motif in the cross-correllation?

- How far out in reciprocal space is there useful data?

- What is a reasonable normalization?

- What origin defining beams should be used?

- What parameters should be used in the genetic analysis to both avoid premature convergence and inadequate searching?

- How should the solutions be searched?

What is the correct/best way to measure the data, i.e. what parameters to use for the motif in the cross-correllation?

The DP analysis creates a square picture then uses the central circular region for measurement, and the region outside of this to measure the background. You need the central region so the peak is fully contained with some background around it. You also need the part outside this circle to have enough pixels for a adequate background subtraction. If the peaks are too close together then you cannot do this well so the measurements are not going to be accurate. You need to adjust the values for (in the “Expert” window, in the main edm the “E” below “Show Ave Peak”

Max Displacement		What displacement from the perfect position to 						accept

XCF Motif Base Level	How small an intensity to use for the motif

Fractional Motif Size		Size of Motif as fraction of reciprocal spacing

Fractional Mask Size		Size of circular mask inside Motif

Threshold			Controls what is considered real – larger values use

less, smaller more. An approximate value is in 						“Info”

Search Radius			Use this so peaks which are just noise are excluded

(See also the next section)

How far out in reciprocal space is there useful data?

Both when measuring the data, and later in the analysis you have to choose how far out to trust the data. In the intensity measurement this is the Search Radius in the Expert Window. When merging data this is in the DataControl file, and for the direct methods this is adjustable by using trim2D and/or manually editing the hkl file. If you go too far out then the values are mainly noise, and do not have any useful information. As the diffraction intensities decay as one goes out in reciprocal space, the signal-to-noise ration will get too small in which case you can have GIGO.

There are number of tools that you can use to judge the accuracy of your data:

- The Motif Window, use “Log Scale” or adjust the Contrast/Brightness.

- “Diff & Symm” in the main window which shows a difference DP (i.e. one with the peaks subtracted). You should check this to see if the spots are being cored (i.e. overmeasured)

- Turn off/on the “Show” in the Lattice section of the window (under “Find”) as well as the “Show Peaks” in the Peaks/Intensity Window. Are you measuring noise?

- Pan the window around to decide if you are going too far out (measuring noise) or not.

- Later (after merging the data) you can look at the raw and averaged values. Are these sensible? Are some spots dynamical (very different intensities between symmetry equivalents?).

What is a reasonable normalization?

There are several options for the normalization you can use. With very good data this does not matter much. With not so good data you should play around with this and the number of beams used in the genetic analysis.

What origin defining beams should be used?

If you check the “Origin” box in Direct Methods the code will try and do this for you, but you can probably do better yourself. Think about what origin definition is, and do it yourself – it is a good exercise.

What parameters should be used in the genetic analysis to both avoid premature convergence and inadequate searching?

If you use too many beams then you can have premature convergence, i.e. it will find one local minimum and not search enough. If you have too few you may not get good enough data. You can adjust this in the main Direct Methods window.

You can also consider setting some reflections yourself. For instance, if in all the useful solutions one (hk) is 360 (e.g. set by a Sigma-1 condition) you can set it yourself then rerun the convergence analysis.

How should the solutions be searched/refined?

This is very, very, very tricky. If you know how many atoms there are (and what they are), then this is relatively simple. If you do not, then you have to think at every step. You may want to use the constraints to stop atoms touching, and the “+” and “-“ with “Use Atoms” to add atoms at test locations. The way to approach this is to think “Maybe it is this – let me test and see if it give a better R-value”.